AMI-2  KIT INSTRUCTIONS

http://www.ilinkboards.com

Be careful not to put too much solder on the connections. Always check for solder bridges. If you make a mistake you may need to buy solder wick to remove the part.

- Check parts against the parts list.

- Place all resistors on the board.

- Place all capacitors on the board.

- Place the Diode on the Board. (Note the direction of  the band)

- Place the Y2 Resonator(3.58 mhz) in its correct position. It looks like a capacitor with 3 pins on the bottom.

- Put the 18 Pin Sockets (U1-U2) on the board. Set them in place such that the end with the opening matches that of the board. This shows what direction the chip will be placed in the socket.

- Place all connectors on the board (5-phono jack sockets, a power plug, the DB9 serial port plug and two stereo jack plugs. Solder all connections including the larger holes. Make sure they line up straight.

- Place the two blue potentiometers on R22 and R15. Carefully line them up and solder them.

- Place the Voltage Regulator (LM340T or 7805) on U3 with it’s back, silver part, facing the LEDS.

- Put the relay and transistors (Q1-Q2) in place. Make sure to follow the board markings.

- Put the LED’s in. D5 (Red-Power), D8 (Green-DTMF), D9 (Yellow-PTT) and D4 (Red-Activity)

- Put WO9G (Bridge Rectifier) in place at D7. The positive side should match the + mark on the diagram and is nearest the J2 Marking on the board.

- Lastly, place the DTMF Decoder (MT8870) in U1 and PIC 16F628 in U2. Observe the correct direction of the chips. (notch to notch)

- The power cable will work in either direction. However, it is best to use center positive. (9-13.5V)

GOOD LUCK and have Fun!! JIM,  WB2REM