AMI-2 Pic Setup Through Hyperterminal

Run hyperterm select the COM port and set it to 2400,n, 8, 1,

The command letters are:

H               Help

?               gives status of the board

R               receive

T               transmit

L               sets the TX timeout

(ie 18=180sec, 3=30 secs)

I               sets the polarity of the COS.

J               sets RTS(Standard) or DTR

COS threshold is fixed at about 1 Volt

In the future Pic Setup software will be available through the AMI-2 Yahoo Newsgroup. Feel free to join the Newsgroup to get updates and communicate with others using the AMI-2.

You can enroll in the AMI-2 Newsgroup at http://www.yahoogroups.com