,

AMI-2 WIRING INSTRUCTIONS

Thank you for your AMI-2 Board order. The wiring directions are listed below.

1. Make up a wiring harness to the microphone connector of your rig. Use a (RS 42-2434) 1/8 male mono audio plug cable (one side bare wire, the other side has the plug). Solder the center wire to the PTT connection and shield to the PTT ground. Likewise use another 1/8 male mono audio plug cable and attach the center to the microphone audio and shield to the microphone ground. Plug the microphone audio and PTT 1/8 Jacks cables into the Radio Mic. and PTT sockets on the back of the AMI-2.

(Note)  If you are using COS connect it to the COS plug on the board and set polarity in the Hyper-terminal setup. Voltage is fixed at 1V.

2. Next obtain THREE – 1/8 to 1/8 plugs (RS) 42-2420 mono audio plug cables.  One end goes into the RX audio out (speaker out) of the radio and the other end into Main RX of the AMI-2 Board. The other cables will go from the sound card speaker and microphone jacks on the back of the AMI-2 to the soundcard speaker and microphone. NOTE: These are the only cables that can be substituted with stereo cables!!! Mono audio cable will also work. The Radio Microphone gain level and Sound Card microphone gain levels can be adjusted through the screw holes on the front panel as well as through the volume control of your radio and sound card adjustments. CAUTION: Use only a small Phillips head screw driver and be careful not to apply too much pressure to the control.

3. As an option you can attach a separate radio receiver to AUX  RX input through another mono audio cable. This is only active when the link is transmitting a signal from the internet and is used to shut down the link as an emergency procedure.

4. Obtain a straight through DB9 Serial Port extension cable. Plug it into the serial port of the computer and into the AMI-2.

5. Download Echolink and run the sysop wizard. Set DTMF commands in EL. Do not use “D”. “D” when hit on the AMI-2 triggers a momentary carrier for distance checking.

6. Attach the power plug. Polarity does not matter although center + is the preferred scheme. When powered up the Red LED will light. If you experience a hum it relates to a possible ground loop. The easiest solution is to use a separate power supply for the AMI-2 then the one used by the rig. The AMI-2 requires a 9-15V supply of at least 500 MA. A cheap Wall Wart works fine.

7. Run the program.  It should start right up. If the software senses the AMI-2 the Red activity LED will flash wildly. If it doesn’t, there maybe a problem with your serial port setup. Go to http://www.ilinkboards.com/echolinksetup.html for help.

PS. If possible, in the information section of your link, could you mention that  you are using a WB2REM AMI INTERFACE (http://www.ilinkboards.com).  THANKS!

73

JIM, WB2REM