AMI

Run hyperterm select the COM port and set it to 2400,n, 8, 1,

The command letters are:

?               gives status of the board

R               receive

T               transmit

L               sets the TX timeout

I               sets the polarity of the COS.

COS threshold is fixed at about 1 Volt

----------------------------------------------------------

BLOCK DIAGRAM

Bill Of Materials        January 31,2003      12:32:10	Page1

Item	Quantity	Reference	Part

______________________________________________

1	7	C1,C2,C3,C4,C7,C8,C9	0.1uF

2	1	C6	.001

3	1	D1	12V Zener

4	2	D3,D2	1N4148

5	2	D5,D4	LED

6	1	D7	BRIDGE

7	2	JP1,JP5	HEADER 2

8	5	J1,J3,J4,J5,J6	PHONEJACK

9	1	J2	POWER SKT

10	1	K1	RELAY DPDT

11	1	P1	CONNECTOR DB9

12	1	Q1	BC337, 2N2222 or any NPN

13	1	R1	2k2

14	2	R2,R12	4k7

15	3	R3,R5,R9	10k

16	3	R4,R6,R7	100k

17	2	R13,R8	100

18	2	R11,R10	470

19	1	R14	1k

20	1	R15	10k VAR

21	1	U1	MT8870/HT9170

22	1	U2	PIC16F628

23	1	U3	7805

24	1	Y2	3.58MHz Resonator