AMI WIRING INSTRUCTIONS

Thank you for your AMI Board order. Please follow the directions below wiring the harness:

- Make up a wire harness. Make up a mike connector to your rig that has wires attached to the PTT, PTT Ground and Microphone Audio & Microphone Ground.  Run the PTT wires to the link board and attach a 1/8 mono male plug.

(Note)  If you are using COS connect it to the COS plug on the board and set polarity in the Hyper-terminal setup. Voltage is fixed at 1V.

2.	Next run the Radio MIC Audio and ground wire with an attached 1/8 mono male 	plug to the Speaker Out of your computer’s sound card. You will eventually         	need to adjust the volume control of the computer.  If you add a 1:1 (RS-273- 	1374) transformer between your Speaker Out and Radio Mic In it could help        	improve the audio quality and reduce hum.

3. Next you will need to buy 2 – 1/8 plug to 1/8 plug (RS) 42-2420  mono chords.  One end goes into the RX audio out and the other end into Main RX of the I-LINK Board. The other chord will go from the PC Mic connector on the I-link board to the Mic in on the computer sound card.

4. You can also do the same for the AUX audio and connect it to a secondary audio source. This is only active when the link is transmitting a signal from the internet.

5. Download Echolink and check sysop. Make sure you put a –L after your call.

6.  Plug a DB9 Serial Port line into the board and computer.

7. Attach the power plug. Polarity does not matter.

Run the program.  It should start right up. The Green LED will flicker on the board and red LED will stay on. If it doesn’t ,there is a problem with your serial port connection or the setup of the ECHOLINK Software. Go to http://www.ilinkboards.com/echolinksetup.html for help.

PS. If possible, in the information section of your link, could you mention that  you are using a WB2REM AMI INTERFACE (http://www.ilinkboards.com).  THANKS!

73

JIM, WB2REM